M

S

L

Mountain Launch System

A white paper on gravity assisted launch

By Keith Watts								August 28, 2009

______________________________________________________________________________

ABSTRACT:

The purpose of this paper is to explain an idea for significantly lowering the cost per pound of launching payloads into space.  Current launch systems rely solely on expensive and complex rocket propulsion to lift payloads into space.  This idea uses inexpensive and plentiful natural resources such as dirt and gravity to improve the performance of existing launch systems.  It uses existing technology and materials.  This idea would also be of great benefit to the countries of Ecuador, South America and Kenya, Africa given their unique geological features.

BACKGROUND/PROBLEMS:

Rocket launches are expensive.  It is an exponential problem with not only fuel required to lift the payload but fuel to lift the fuel.  Lifting one pound into orbit requires 34 to 60 lbs of fuel.  Most of this fuel is used in the first few seconds of flight, lifting a stationary rocket and payload from sea level and accelerating through the thick lower atmosphere.  In the case of the space shuttle in the first 8 seconds, it has traveled 300 ft and consumed half of its fuel supply (1.5 million pounds).  My idea came from thinking about using what we have in abundance like dirt and gravity and low cost materials such as steel and concrete and applying these resources to the problem of rocket launch.

In addition, most of the world launch sites are located in coastal areas near sea level.  Rockets must push through the thickest part of the atmosphere and travel the furthest distance to space.  They are also vulnerable to forces of nature, and possible attack.

SOLUTION/THE IDEA:

The basis of the idea is quite simple, that is to use a counterweighted platform similar to early elevators (see figure 1).  Rather than use a single counterweight the idea uses multiple counterweights perhaps 10-12 arranged like numbers on a clock face around the central launch shaft (see figure 2).  The limit of a manmade structure would be approximately 1000 ft, similar to a 100 story building and probably not worth the effort.  What is needed is a really large natural structure, a large pile of dirt, fortunately such large piles of dirt exist, they are called mountains.  Hence the name of this idea is the Mountain Launch System or MLS.

In terms of gravity, we usually think of free falling, but instead think of free ascending.  When a skydiver freefalls, he quickly reaches a terminal velocity (120 mph) where the forces of aerodynamic drag and the gravitational pull balance each other and he no longer accelerates.   Because the MLS is located inside of a mountain nearly all of the surface area is sealed.  Thus if you put a hatch on the bottom of the launch tube and a thin membrane (think cellophane) over the top, then the air can be evacuated from the system.  With the air removed there is no aerodynamic drag and hence no terminal velocity.   Acceleration will continue proportional to the length of the tube.  See figure 3.

Main launch tube

Counter weight

shafts 12 places

FIGURE 1					       FIGURE 2

Early counterweight elevator			Cross section of MLS

Vacuum pump

Mountain

Launch tube

Thin membrane

Lower hatch

Rocket

FIGURE 3

Evacuation of air from system

HOW BIG IS IT?:

Current Atlas and Delta rockets weigh approximately 500,000 lbs, using a 4:1 ratio this would require 2,000,000 lbs of counter weight.  The main launch tube would be 6 meters in diameter and the 12 counterweight tubes could be 1 meter in diameter.  Each tube would then contain a 150,000 lb counterweight.  Wire ropes of 1 inch in diameter have a 100,000 lb rated load and weigh 10,000 lbs, so a minimum of 2 would be required.  The resulting acceleration on the rocket is axial and less than 1 G so the rocket structure should be adequate.  This would allow the use of existing launch vehicles.  The counter weights would be winched up individually before being attached to the launch platform.  No exotic technology is required just steel and concrete.

THE NUMBERS:

Using the 4:1 counterweight ration mentioned above the resulting vertical acceleration would be ¾ G going up.  For a 10,000 ft launch tube, in 29 seconds the rocket would exit the top of the tube traveling at 475 mph!  For a 20,000 ft launch tube it would take 41 seconds and the speed would be 670 mph!  This means the rocket would be above 10,000 ft and traveling at nearly 500 mph without having consumed any onboard propellant!

MOUNTAINS ON THE EQUATOR?:

The advantages of launching near the equator are well known and it so happens there are two large mountains (greater than 15,000 ft) located on the equator (marked by red X) where the Mountain Launch System would be of the largest benefit.    There are also smaller (approx 5,000 ft) mountains in Malaysia, see figure 4.

X

X

FIGURE 4

Mountains on the equator

Mt Chimborazo, Ecuador

This is located on the west coast of South American near Quito which is the capital of Ecuador.  It is 20,560 ft in elevation and has the distinction on being the outer most point on the planet.  Mt Everest is higher, but due to the equatorial bulge of the Earth, Mt Chimborazo is the point furthest from the center of the Earth.  It is a stratovolcano, which last erupted in 640AD +/- 500 years. The coordinates are 1°28′9″S, 78°49′3″W.  Over flight of South America is a concern with this location as well as recent seismic activity in the area.  See figure 5.

FIGURE 5

Mt Chimborazo, Ecuador

Mt Kenya, Kenya

This location is near the West coast of Africa not far from the old San Marcos launch platforms.  It is an extinct stratovolcano which last erupted 2.6 to 3.1 million years ago.  The nearest city is Nairobi.  It rises to an elevation of 17,058 ft and has coordinates of 0°9′0″S, 37°18′0″E.  The over flight concern is minimal as it is not far from the coast and the area is not heavily populated.  This location is well suited to serve a large percentage of the world’s population, located close to Europe and Asia.  A world space port in Kenya would provide the country and continent with a great deal of pride and prestige in the 21st century.  See figure 6.

FIGURE 6

Mt Kenya, Kenya

USE WITH RAMJET:

For even greater efficiency to orbit, the speeds achieved by the MLS would be sufficient to ignite a ramjet which would consume atmospheric oxygen up to an altitude of 100,000 ft.   A conventional rocket with on board propellant would be used for the remainder of the mission, perhaps an existing 3rd stage rocket.

MORE POWER!:

Greater launch speeds are achievable by adding power to the counterweight pulleys.  The acceleration would need to be gradual depending on the structural capability of the rocket.  Magnetic drive could also be employed to increase the speed of ascent.

SHIELDED LOCATION:

Current launch facilities suffer delays due to weather; recent examples include hail damage to the Shuttle and hurricane damage to buildings.  They are also vulnerable to a military or terrorist attack.  The MLS located inside of a mountain is naturally shielded and easier to defend.  It would be a method to provide assured access to space.

DESIGN DETAILS:

The membrane on top of the launch tube could be pierced by the rocket fairing or burned through with a heater before the rocket arrived.

Regenerative braking could be used to slow the counterweights after the rocket launched to recapture a large amount of energy.

OTHER USES:

MLS could be adapted for use in micro gravity research by providing a controlled free fall environment.  With the increasing interest in space tourism a low cost vehicle with solid boosters could carry paying passengers to the edge of space.

HOW TO BUILD IT:

I would expect the approach to be from the bottom up.  Tunnel in horizontally and create a large central chamber.  Tunneling up could be done in sections with a construction platform being raised by counterweights.

WILL IT WORK?:

I cannot say with certainty that this idea will work.  There will be financial, political and technological obstacles to overcome.  I think the Sea Launch system is similar example of an unconventional approach that has proved successful.  Certainly there is the potential for improving the performance of existing rockets which has been difficult recently.

NEXT STEPS:

- Preliminary design concepts

- Consultation with geologists on feasibility

- Consultation with civil engineers on feasibility

- Possibly a technology demonstrator in the Owens valley.  Use of a Cliffside or an abandoned oil well for a proof of concept.

- Use with small missiles, ramjets or rockets and work up to larger sizes

CONCLUSION:

I wish to thank the reader for taking the time to learn about the Mountain Launch System.   The MLS can take existing rockets and accelerate them to high speed and give them a boost to orbit.  This will result in improved payload performance and a lower cost per pound to orbit.  The MLS uses conventional materials such as steel and concrete and no new technology is required.  We are seeing the start of the space tourism industry with a lot of new rocket development.  This idea does not compete with these systems but rather provides a means for further increasing their performance.

THE AUTHOR:

Keith Watts has worked in the Aerospace industry for the past 30 years.  The first 8 years was in aircraft structural design and the last 22 years has been in satellite structural design and system engineering.   He is currently is an Associate Tech Fellow with Boeing in El Segundo CA

Contact information:

Email: keith.p.watts@boeing.com

Phone 310-364-5666

PATENT No: 7530532, issued May 12, 2009